## Training Stages

Stage 0: all ports lights on; reward always available (with bias correction on); 50 trials

Stage 1: dot projected (north and centre locations); port lights on after dot offset

- Stage 1.0: reward available for 30s after dot offset; dot trigger time 50ms; 40 trials

- Stage 1.1: reward available both ports 0 and 1 on every trial for 15s after dot offset; 40 trials

- Stage 1.2: port lights off; 40 trials

- Stage 1.3: dot trigger time 100ms; reward available for 10s after dot offset; 40 trials

- Stage 1.4: dot trigger time 200-300ms; 40 trials

Stage 2: sound on after dot offset; no switching allowed

Stage 3

- Stage 3.0: Add additional dot locations

- Stage 3.1: All dot locations

Stage 4: Audio cue is triggered as the mouse enters a ROI or diameter from dot location

- Stage 4.0: small sound ROI around dot

- Stage 4.1: larger sound ROI around dot; reward available for 5s after dot offset

Stage 5.0 (final): sound starts playing at t=∆t before dot projection

- Essential features:

  - Introduce sound at fixed time Δt before presentation of dot, (where Δt is parameterised).

  - Two timeout conditions should be applied: 1) timeout between dot projection and offset, 2) timeout between dot offset and nose poke. Cause of timeout should be logged in .csv file.

  - Option to parameterise % trials of type A and % trials of type B, either in an external csv or from bonsai (either way this should be logged).

- Nice to have:

  - Option to alternate between blocks of trial type A and trial type B, where blocks of length n trials (where n is parameterizable).

  - Option to offset sound at fixed time t after dot offset. (This could be an added feature of trial type B, so if t is set to 0 we have the original type B).

Stage 7.0 (single location test): only port 0 or port 1 dispense a reward. We can adjust the size of the trial block and what context plays

- Essential features:

  - Present rewards only on port 0 or only on port 1 regardless of audio cue

  - Option to specify % trials sound A, % trials sound B, % trials no sound where the reward port identity alternates between blocks (between port 0 and port 1)

- Nice to have

  - Alternating blocks of length n trials, where

## Callibration workflow(s)

Valve calibration

Flush valves

Calibrate dot locations

## Global features

Workflow should be working across all boxes

- Refactored workflow should be the same between boxes, differing only in parameters such as calibration (which must vary between boxes to keep a consistent coordinate frame).

- Option to set / save box-specific parameters locally without issues with github. E.g. box-specific parameters could be set in a .csv file which is on the repo for all boxes?

- All conditions should be the same between all boxes, e.g. audio cue pitch / volume, dot brightness, location of dots relative to ports, so that we can smoothly transition animals to the ephys box

Global task logic features

- Beep on top offset: Pure tone of duration ~200ms

- Antibias algorithm: Implement Antibias algorithm over a sliding window of  length w based on choice and reward history (exact algorithm TBD), ideally such that the kernel of the sliding window is weighted towards more recent trials.

- Exclude dot projection within radius r1 from mouse: Dot should never be projected on top of the mouse so he doesn’t accidentally trigger it

- Option to automatically progress through the stages after n correct trials (set True/False)

- Dot offset should be triggered by nose poke to dot (rather than distance from centroid)

Wall rotation control

- Wall rotation = true → no feedback after each trial (no reward or timeout after touching the port). All task parameters, e.g. choice of port should be recorded as normal.

- Walls rotated = true → log whether or not walls were rotated as one of the session parameters. (No change to task logic).

User-specified parameters

- Dot variables

  - Dot onset radius

  - Dot brightness

  - Dot diameter

- Audio variables

  - Audio attenuation for each audio cue

  - Mapping from audio cue identity to correct port

- Data output folder

- Path to relevant .csv files for bonsai workflow parameters

Nice to have:

- Option to project dot within maximum distance r2 :

## Data logging

- All user-specified parameters (global and stage-specific) should be automatically logged and saved for each session, including:

  - mapping from audio cue identity to port location

## Interface

User-specified parameters set in GUI

Visualiser:

| Mouse ID | 101 |
| --- | --- |
| Stage | 4.1 |
| Time since experiment started (min) | mm.ss |
| Reward size (uL) | 2 |
| Trial number | 300 |
| Correct Count | 250 |
| Total volume of reward (uL) | 500 |
| Rolling accuracy |  |
| Rolling bias |  |